# Freenove 4WD Car Challenges

# Guide How to Download

- In the Arduino IDE, go to Sketch -> Export Compiled Binary.

- Open your sketch folder (Ctrl+K). You will see a file ending in .uf2.

- Drag and drop that .uf2 file directly into the RPI-RP2 drive.

- The Pico will automatically reboot and start running your code.

If you cant see the Pi on your PC then..

- Unplug the Pico from your computer.

- Press and hold the BOOTSEL button on the Pico board.

- While holding the button, plug the USB cable back in.

- Release the button. Your computer should now show a new drive named RPI-RP2.

# Lesson 1 – Car Move & Turn

🚦 The Basics: How to Read the Code

The "Magic Command": Motor_M_Move(LeftFront, LeftRear, RightFront, RightRear);

- Positive numbers (1 to 100): Wheel goes forward.

- Negative numbers (-1 to -100): Wheel goes backward.

- delay(1000): Wait for 1000 milliseconds (which is 1 second).

🏆 Challenge 1: The Speed Racer

The Goal: Make the car go twice as fast when moving forward, and super slow when moving backward.

- How-to: Look at the first Motor_M_Move line. Change the 50s to a higher number (up to 100).

- Tip: Ask them: "If 100 is the fastest, what number would make the car crawl like a turtle?"

🕒 Challenge 2: The Long Haul

The Goal: Make the car drive forward for 3 seconds instead of just 1.

- How-to: Find the delay(1000); line right after the forward command. Change the number inside the parentheses.

- Tip: Remind them that the computer counts in milliseconds. 1000 = 1 second, so 3000 = 3 seconds.

🔄 Challenge 3: The "Dizzy" Spinner

The Goal: Make the car spin in a circle in one spot (a "Zero Turn").

- How-to: To spin, one side of the car needs to go forward while the other side goes backward.

- The Change: Try Motor_M_Move(50, 50, -50, -50);.

- Tip: If the car just turns slightly, increase the delay so it keeps spinning longer!

# Lesson 2 – Matrix Display

Those two squares on the front of your car are like the robot’s "eyes," and you get to tell them exactly what to do. Right now, your robot is programmed to blink, but you can change its personality just by tweaking the code.

Here are 5 missions for you to try!

🚀 Mission 1: The Fast-Blinker

The robot is currently blinking slowly. Let’s make it look like it just drank a giant soda!

- Find this line: showArray(500); (It’s at the bottom in the void loop).

- The Goal: Change 500 to 100.

- The Secret: That number is "milliseconds." 1000 is one second. By making the number smaller, you’re telling the robot to move faster!

😉 Mission 2: The Pirate Wink

Can you make the robot wink one eye while keeping the other one wide open?

- Look at: y_array (that’s the right eye).

- The Goal: Copy the first line of numbers and paste them over the second line so they are exactly the same.

- The Secret: If the right eye stays the same but the left eye (x_array) keeps changing, it will look like a wink!

🌙 Mission 3: Night Mode

The robot’s eyes might be really bright. Let’s dim them down so it can "see" in the dark without hurting its eyes.

- Find this: matrix.init(0x71); (inside void setup).

- The Goal: Right under that line, add this: matrix.setBrightness(1);

- The Secret: You can use any number from 0 (off) to 15 (super bright). Try 15 to see the difference!

😠 Mission 4: Angry Robot

Let’s make the eyes look like flat, grumpy lines.

- Look at: x_array and y_array.

- The Goal: Change one of the rows of numbers to this: 0x00, 0x00, 0x00, 0xFF, 0xFF, 0x00, 0x00, 0x00

- The Secret: 0x00 means "lights off" and 0xFF means "all lights on in this row." You just drew a thick line in the middle!

🎨 Mission 5: The Master Artist (Custom Face)

This is the hardest mission! Can you make the eyes look like Diamonds? 💎

- The Goal: Use these "Secret Codes" for your array: 0x08, 0x1C, 0x3E, 0x7F, 0x7F, 0x3E, 0x1C, 0x08

- The Secret: Each of those codes represents one row of the 8x8 grid. If you put those in, the robot will have diamond-shaped eyes.

💡 Top Secret Coding Tips:

- The "Comma" Rule: Make sure every number has a comma after it, except for the very last one in the row. If you forget a comma, the robot gets a "headache" and won't run!

- The "Zapped" Rule: If you make a mistake and the code won't upload, don't worry! Just hit the "Undo" button (Ctrl+Z) to go back to when it worked.

- The "Drawing" Trick: If you want to draw your own face, ask a grown-up to find an "8x8 LED Matrix Tool" online. You click the dots, and it gives you the 0x codes for your robot!

# Lesson 3 Ultrasonic Ranging

🛰️ The "Head" and the "Ears"

The robot is using two main parts for this:

- The Servo: This is a little motor that acts like a neck. It can turn to specific angles (like 30°, 90°, or 150°).

- The Ultrasonic Sensor: These are the two round things on the front that look like eyes, but they are actually ears! They send out a "ping" of sound and listen for it to bounce back to see how far away an object is.

📝 What the Code is Doing

When you run this program, here is the robot's "To-Do" list:

1. The Startup (setup)

- It wakes up the Serial Port so it can talk to your computer screen.

- It gets the sensor and the "neck" (servo) ready.

- It looks straight ahead by setting the angle to 90°.

2. The Scouting Loop (loop)

The robot will do these four steps over and over again forever:

- Look Left: It turns its head to 150° and "pings" to see how far away things are on the left.

- Look Center: It turns back to 90° and checks the distance straight ahead.

- Look Right: It turns its head to 30° and checks the distance on the right.

- Look Center again: It turns back to 90° to finish the scan.

🚀 Mission 1: The Fast Scanner 🏃‍♂️

The robot is currently scanning quite slowly. Let's see if you can make it "watch" the room like a fast-moving security camera!

- The Goal: Make the robot look left, center, and right much faster.

- The How-to: Find all the delay(500); lines. Change that 500 to 200.

- The Secret: Lowering the delay makes the robot wait less time between movements. If you go too low (like 50), the robot’s neck might get a bit shaky!

🚀 Mission 2: The Owl Turn 🦉

Owls can turn their heads really far around. Your robot is currently only looking a little bit to the left and right.

- The Goal: Make the robot look as far left and as far right as it possibly can.

- The How-to: Change the angle 150 to 180 (Maximum Left) and change the angle 30 to 0 (Maximum Right).

- The Secret: Servos usually move from 0 to 180. 90 is always the middle!

🚀 Mission 3: The Wall Alarm 🚨

Let's use the Serial Monitor on your computer to play a game of "Don't Touch!"

- The Goal: Use your hand to make the robot "yell" at you through the computer screen when you get too close.

- The How-to: Open your Serial Monitor. Look at the distances. Try to find a way to make the robot say "TOO CLOSE!" when the distance is less than 10.

- The Hint: This is a bit of a trick! You'll need to use an if statement. If that's too hard yet, just try to move your hand until the Serial Monitor shows exactly 5. It’s harder than it looks!

-   Tip 1: The "If" Secret: To make the robot think, we use an if statement. It looks like this: if (Get_Sonar() < 10) { Serial.print("TOO CLOSE!"); } This tells the robot: "If the distance is less than 10, shout at me!"

-   Tip 2: Where to Put It: Place your "if" code right after the Get_Sonar line. That way, the robot checks the distance and immediately decides if it needs to yell.

-   Tip 3: The Ghost Numbers: Sometimes the sensor sees "0" or a huge number for a split second if the sound wave bounces weirdly. Don't worry if the alarm goes off once by mistake—that’s just "sensor noise!"

🚀 Mission 4: The "Yes" Man ✅

Can you make your robot shake its head "Yes" instead of "No"?

- The Goal: Change the code so the robot looks up and down instead of left and right.

- The How-to: Actually... check your robot! Is the sensor mounted to look side-to-side or up-and-down?

- The Secret: If your servo is mounted sideways, you can't make it look "up." To finish this mission, you have to re-write the code so it only uses two angles: 70 and 110, switching back and forth fast!

-   Tip 1: The Small Shake: If your robot's head moves side-to-side, a "nod" is just a very small, fast movement. Instead of going from 30 to 150, try just going from 80 to 100. It will look like the robot is shivering or saying a quick "yes!"

-   Tip 2: Speed is Key: To make it look like a real nod, you need to change the delay. Try delay(100);. If the delay is too long, it looks like a slow turn. If it’s short, it looks like a quick gesture!

-   Tip 3: Check the Neck: Before you hit upload, move the robot's head gently with your hand. Does it move easily? If it feels stuck, don't force it—just make sure your code stays within the numbers that feel "smooth."

🚀 Mission 5: The Guard Dog 🐕

A guard dog stays still until it sees something move.

- The Goal: Make the robot stay at 90 degrees (looking straight) for a long time, then do one quick "scan" (left and right) and go back to waiting.

- The How-to: At the very end of your void loop(), add a giant delay: delay(5000);.

- The Secret: 5000 is 5 seconds. Your robot will look around, then "stare" straight ahead for 5 seconds before checking its surroundings again.

  Tip 1: The "Loop" Order: Remember, the robot reads the void loop like a book—from top to bottom. If you put the delay(5000) at the very bottom, it will finish its whole scan before it "takes a nap" for 5 seconds.

  Tip 2: The Statue Move: If you want it to look even more like a guard dog, make sure your very last command before the long delay is Servo_1_Angle(90);. This ensures it’s looking straight ahead while it waits.

  Tip 3: Change the Nap Time: If 5 seconds feels too long, you can change the "nap" whenever you want.

- 2000 = 2 second nap.

- 10000 = 10 second nap!

💡 Engineering Pro-Tips:

- The "90" Rule: Whenever you get confused about where the robot is looking, set the angle to 90. That is the "Home" position.

- Watch the Wires: When you make the robot turn to 0 or 180, make sure the wires don't get tangled or pulled too tight.

- The Serial Monitor: Remember to click the little Magnifying Glass or Serial Monitor icon in your coding program, otherwise, you won't be able to see the distance numbers!

## Lesson 4 – IR Follow

Think of this code as the "Brain" of your robot. It follows a simple loop: Look, Decide, Act.

Here is the secret behind how it thinks:

1. The "Eyes" (Reading the Sensors)

The robot has three sensors on the bottom. In the code, Track_Read() tells the robot to look at the ground. It stores what it sees as a "Sensor Value."

- 0 means it sees the white floor.

- 1 means it sees the black line.

Because there are three sensors, they combine into a 3-bit "secret code" (like 010). The computer turns that code into a single number (0 through 7) so it can make a quick decision.

2. The "Decision" (The Switch Case)

The switch section is like a giant list of rules. The robot asks: "Which number did my eyes just see?"

- If it's 2 (010): The line is right in the middle! Rule: Drive straight.

- If it's 4 or 6 (100 or 110): The line is sliding to the left! Rule: Turn left to get back on it.

- If it's 1 or 3 (001 or 011): The line is sliding to the right! Rule: Turn right.

- If it's 7 (111): It sees a big black wall or finish line! Rule: Stop moving.

3. The "Legs" (The Motors)

Once the brain decides what to do, it uses Motor_M_Move to tell the wheels how to spin.

- Positive numbers make the wheels go forward.

- Negative numbers make them go backward.

- To turn left, the robot actually makes the left wheels go backward and the right wheels go forward—just like how you spin a 360 on a skateboard or a swivel chair!

4. The "Face" (The Emotions)

While the legs are moving, the code uses commands like showArrow or eyesBlink. This doesn't help the car drive, but it tells you what the car is thinking! If it sees the line and shows an arrow, you know the "Brain" is working correctly.

Basically: The robot spends its whole life asking "Where is the black line?" and "How fast should I spin my wheels to stay on it?" millions of times every second!

/**********************************************************************

Filename    : Motor_Only_Line_Follower.ino

Description : Pure line following logic using clean binary masking.

Wheel Order : Front Left, Back Left, Back Right, Front Right

**********************************************************************/

#include <Arduino.h>

#include "Freenove_4WD_Car_For_Pico_W.h"

// --- THE PICTURE DICTIONARY (Using True Binary 0b) ---

#define B000  0b000    // All eyes see WHITE

#define B001  0b001    // Right eye sees BLACK

#define B010  0b010    // Middle eye sees BLACK

#define B011  0b011    // Middle + Right see BLACK

#define B100  0b100    // Left eye sees BLACK

#define B101  0b101    // Left + Right see BLACK (Your 101 case)

#define B110  0b110    // Left + Middle see BLACK

#define B111  0b111    // All eyes see BLACK

void setup() {

Track_Setup();

Motor_Setup();

}

void loop() {

Track_Read();

// Combine 3 digital sensor readings into a single 3-bit number

// Left is bit 2, Middle is bit 1, Right is bit 0

int picture = (sensorValue[0] << 2) | (sensorValue[1] << 1) | sensorValue[2];

// --- THE MOTOR RULE BOOK ---

switch (picture) {

case B000: // Lost (All White - Spin/Search)

Motor_M_Move(-40, -40, 40, 40);

break;

case B111: // All three eyes (Straight)

Motor_M_Move(0, 0, 0, 0);

break;

case B010: // Middle eye only (Straight)

Motor_M_Move(30, 30, 30, 30);

break;

case B101: // Left + Right eye (Go Straight)

Motor_M_Move(0, 0, 0, 0);

break;

case B100: // Only Left eye (Steer Left)

Motor_M_Move(-40, -40, 40, 40);

break;

case B110: // Left and Middle eyes (Steer Left)

Motor_M_Move(-40, -40, 40, 40);

break;

case B001: // Only Right eye (Steer Right)

Motor_M_Move(40, 40, -40, -40);

break;

case B011: // Middle and Right eyes (Steer Right)

Motor_M_Move(40, 40, -40, -40);

break;

}

}

Challenge 1: The "Top Speed" Test

- Make the robot go around the track as fast as possible.

Challenge 2: Reverse

- Make the robot reverse backwards in a straight line whenever it loses the line.

Challenge 3: The "Backwards"

- Make the robot go around the track backwards.

# Lesson 5

Lesson 5 – Obstacle Avoidance (Bat Sonar!)

Think of this code as your robot’s "Spidey-Sense." Instead of looking at the floor, it is now looking straight ahead to make sure it doesn't crash into a wall, a chair, or the family dog! It follows a new loop: Shout, Listen, React.

Here is the secret behind how it avoids crashing:

1. The "Bat Ears" (The Ultrasonic Sensor) The robot has a piece on the front that looks like two big eyes, but they are actually a tiny speaker and a microphone! In the code, a command like readUltrasonic() tells the robot to do exactly what bats and submarines do:

- The Shout: It sends out a super-high invisible squeak (too high for our human ears to hear).

- The Listen: It waits to hear the echo bounce back off a wall.

2. The "Math Brain" (Calculating Distance) The computer uses a built-in stopwatch to count how fast the echo comes back.

- A short time means the wall is right in front of it!

- A long time means the path ahead is totally clear. The code does a quick math trick to turn that time into a simple number you can read, like "15 centimeters."

3. The "Decision" (The If/Else Block) Instead of a giant switch list, this code usually uses a simple If/Else rule to keep the robot safe. The robot constantly asks: "Am I about to bump my nose?"

- If the distance is less than 15 cm: Danger! Rule: Hit the brakes immediately.

- Else (if it's greater than 15 cm): The coast is clear! Rule: Keep driving forward.

4. The "Escape Plan" (The Motors) When the robot gets too close to a wall, stopping isn't enough. It has to escape! It uses the Motor_M_Move block to perform a special combo:

- First, both wheels get negative numbers to back away from the wall.

- Next, one wheel goes forward and the other goes backward to spin the robot around.

- Now, it faces a brand new direction and can go back to exploring!

Basically: The robot spends its whole life shouting into the room and listening for echoes so it never stubs its robotic toe!

/**********************************************************************

Filename    : Photosensitive_Sensor.ino

Product     : Freenove 4WD Car for Raspberry Pi Pico (W)

Auther      : www.freenove.com

Modification: 2023/04/13

**********************************************************************/

#define Left_PHOTOSENSITIVE_PIN 28  //Define the pins that Raspberry Pi Pico reads photosensitive

#define Right_PHOTOSENSITIVE_PIN 27 //Define the pins that Raspberry Pi Pico reads photosensitive

int getLeftPhotosensitiveADCValue;  //Define a variable to store the left photosensitive adc value

int getRightPhotosensitiveADCValue; //Define a variable to store the right photosensitive adc value

void setup()

{

pinMode(Left_PHOTOSENSITIVE_PIN, INPUT);//Configure the pins for input mode

pinMode(Right_PHOTOSENSITIVE_PIN, INPUT);//Configure the pins for input mode

Serial.begin(115200);          //Initialize the serial port and set the baud rate to 115200

}

void loop()

{

getLeftPhotosensitiveADCValue = analogRead(Left_PHOTOSENSITIVE_PIN);//Read ADC1 value value

getRightPhotosensitiveADCValue = analogRead(Right_PHOTOSENSITIVE_PIN);//Read ADC2 value value

Serial.print("The photosensitive ADC on the left: ");

Serial.println(getLeftPhotosensitiveADCValue);       //Print the photosensitive ADC value on the left

Serial.print("The photosensitive ADC on the right: ");

Serial.println(getRightPhotosensitiveADCValue);      //Print the photosensitive ADC value on the right

delay(500);

}

Level 1: The Speed Hacker (Warm-up)

Right now, the robot’s "brain" is reading the light sensors and printing the numbers to the screen twice every second.

- The Mission: "Change the code so the robot reads the light super fast—ten times a second! Then, change it so it only reads the light once every two seconds."

- What this teaches: Understanding the delay() function and how milliseconds work (1000 ms = 1 second).

- The Solution: He needs to find delay(500); and change it to delay(100); (for fast) and delay(2000); (for slow).

Level 2: The Light Detective (Logic & If/Else)

Instead of just spitting out raw numbers, let's make the robot actually understand the numbers.

- The Mission: "Add an if/else statement inside the loop(). If the left sensor's number is bigger than the right sensor's number, tell the computer to print: "The light is on the Left!" Otherwise, print "The light is on the Right!" "

- What this teaches: Comparing two variables using the greater-than > operator.

- The Hint: He will need to add something like this before the delay:

C++

if (getLeftPhotosensitiveADCValue > getRightPhotosensitiveADCValue) {

Serial.println("The light is on the Left!");

} else {

Serial.println("The light is on the Right!");

}

Level 3: The Dark Room Alarm (Variables & Math)

Let's see if he can create a brand new variable and use it to trigger an alarm.

- The Mission: "Create a new int variable inside the loop called totalLight. Make it equal to the left sensor PLUS the right sensor. If totalLight drops below a certain number (like 1000), print "WARNING: IT IS TOO DARK!""

- What this teaches: Variable declaration inside a scope, basic arithmetic (+), and threshold logic.

- The Hint: ```cpp int totalLight = getLeftPhotosensitiveADCValue + getRightPhotosensitiveADCValue;

if (totalLight < 1000) { Serial.println("WARNING: IT IS TOO DARK!"); }